Image Grading Logic & Parameters

This document explains how the 
sort_images.exe tool decides whether an image is "Good" or "Bad".

The "Veto" System (Hard Filtering)

The classification uses a strict veto system. This means an image starts with a "Perfect" grade, and any single flaw causes it to fail immediately. There is no weighted average (e.g., a very sharp image is still rejected if it has digital zoom).

Rule: If ANY parameter fails its threshold, the image is BAD. Rule: The image must pass ALL checks to be GOOD.

Parameters & Thresholds

Here is the breakdown of each parameter, its default weight/threshold, and the reasoning.

1. Resolution (Metadata)

- What it is: The pixel dimensions of the image (Width x Height).

- Threshold: Must be at least 3000 x 2000 pixels.

- Reasoning: Images below this size lack the detail required for high-quality inspection or mapping. They are often accidental low-res snapshots or thumbnails.

- Adjustment: Modify MIN_WIDTH and MIN_HEIGHT in the script.

2. Digital Zoom (Metadata)

- What it is: The ratio of digital magnification applied by the camera.

- Threshold: Must be <= 1.0 (No digital zoom).

- Reasoning: Digital zoom is just "cropping and stretching." It destroys pixel data and resolution integrity. We only want optical zoom or no zoom.

- Adjustment: Modify MAX_DIGITAL_ZOOM in the script.

3. ISO Sensitivity (Metadata)

- What it is: The sensor's sensitivity to light.

- Threshold: Must be <= 1600.

- Reasoning: High ISO (e.g., 3200, 6400) introduces significant "noise" or grain, making the image look speckled and reducing clarity.

- Adjustment: Modify MAX_ISO in the script.

4. Blur Score (Visual Analysis)

- What it is: A calculation of the "Laplacian Variance".

  - High Variance = Many sharp edges = Sharp Image.

  - Low Variance = Few sharp edges = Blurry/Flat Image.

- Threshold: Must be >= 100.0.

- Reasoning: This detects motion blur (drone moving too fast) or focus issues.

- Adjustment: Modify MIN_BLUR_SCORE in the script.

  - If Good images are being rejected: Lower this number (e.g., to 50.0).

  - If Bad/Blurry images are being accepted: Raise this number (e.g., to 200.0).

Example Grading Scenarios

| Image | Resolution | Zoom | ISO | Blur Score | decision | Reason |
| --- | --- | --- | --- | --- | --- | --- |
| Img_A | 4000x3000 (Pass) | 1.0 (Pass) | 100 (Pass) | 500.0 (Pass) | GOOD | All Pass |
| Img_B | 1920x1080 (Fail) | 1.0 (Pass) | 100 (Pass) | 600.0 (Pass) | BAD | Low Resolution |
| Img_C | 4000x3000 (Pass) | 2.0 (Fail) | 100 (Pass) | 300.0 (Pass) | BAD | Digial Zoom |
| Img_D | 4000x3000 (Pass) | 1.0 (Pass) | 800 (Pass) | 40.0 (Fail) | BAD | Blurry |